- Install utildll.exe

- Connect  “USB debug Adapter” to USB port in PC and connector on the LB board in the electronic unit labelled “JTAG_020”

- Power the electronic unit on.

- Run “FLASH Programming Utility” on PC:

Start – All Programs - Silicon Labs - FLASH Programming Utility

- Select in FLASH Programming Utility:

  - “Debug interface” = “JTAG”

  - “Debug Adapter” = “USB Debug Adapter”

  - Press “Connect”

If label “Device name” (on bottom of the panel) will write “C8051F020”. It’s OK.

- Select:

  - “Download HEX File/Go/Stop”

  - “Download Filename” press “Browse”.

  - Select file “Firmware.hex”

  - Select “Erase all Code Space before download”

  - Press “Download”

- When download complete – Power off the electronic unit;

- Close “FLASH Programming Utility”

- Disconnect “USB debug Adapter”.

- Power on the electronic unit.

- If you can see at display device "Tension = x.xx".

Close this file.